# COAL – EXAM ANSWERS PACK (DMA, I/O METHODS, CU TYPES, CHANNEL I/O)

## 1. HOW DMA WORKS (EXAM ANSWER)

DMA (Direct Memory Access) allows I/O devices to transfer data directly to/from memory without CPU copying each byte. This increases system speed.

Step-by-step DMA process:

1. CPU sets DMA controller: source, destination, number of bytes.

2. CPU gives ‘Start DMA’ command.

3. DMA controller takes control of system bus.

4. DMA transfers data directly between memory and device.

5. DMA sends an interrupt to CPU when finished.

6. CPU resumes normal execution.

CPU ---- SETUP DMA ----> DMA Controller ----> I/O Device
                                  |
                                  v
                               MEMORY

Real Example (LMS): Uploading a 2GB video lecture uses DMA to transfer data directly to RAM while CPU handles user sessions.

## 2. DIFFERENCE BETWEEN PROGRAMMED I/O, INTERRUPT I/O, DMA

PROGRAMMED I/O (Polling):

• CPU repeatedly checks device.
• Wastes CPU time.
• Good for simple/slow devices.

CPU -> device ready?
CPU -> device ready?
CPU -> device ready?

INTERRUPT-DRIVEN I/O:

• Device sends interrupt when ready.
• CPU does not waste time.
• ISR handles device.

DEVICE ---- IRQ ----> CPU ----> ISR ----> Resume

DMA:

• CPU is not involved in data transfer.
• Fastest method.
• Best for large block transfers.

DEVICE <----> DMA <----> MEMORY

Short Comparison Table:

• Programmed I/O = CPU busy waiting.
• Interrupt I/O = CPU free until device ready.
• DMA = CPU free + high-speed bulk transfer.

## 3. MICROPROGRAMMED vs HARDWIRED CONTROL UNIT

HARDWIRED CONTROL UNIT:

• Implemented using gates & flip-flops.
• Very fast execution.
• Difficult to modify.
• Used in RISC designs.

Opcode --> Hardware Logic --> Control Signals

MICROPROGRAMMED CONTROL UNIT:

• Stores microinstructions in control memory.
• Easy to change/patch.
• Slower than hardwired.
• Used in CISC CPUs.

Opcode --> Microprogram Memory --> Microinstructions --> Control Signals

Exam Comparison:

• Hardwired = fast, fixed, hardware logic.
• Microprogrammed = flexible, easy to update, slower.

## 4. CHANNEL I/O (DETAILED EXAM ANSWER)

Channel I/O uses special processors called channels to manage I/O devices without involving CPU directly.

Types of Channels:

1. Selector Channel – handles one high-speed device at a time.
2. Multiplexor Channel – handles multiple slow devices simultaneously.

CPU
 |
 |----> CHANNEL PROCESSOR ----> Device 1
                       |----> Device 2
                       |----> Device 3

How Channel I/O works:

1. CPU sends Channel Command Word (CCW) to channel.
2. Channel performs entire I/O sequence.
3. Channel uses DMA to move data to memory.
4. Channel interrupts CPU when done.

Real Example:

Bank servers, university servers using large storage arrays use channel I/O to manage thousands of records efficiently.